# Assignment 1 — Branch Prediction

Al-Akhir Nayan

PSID: 2371302

Pentium M direction predictor

Programming assignment report

## Overview

For this assignment I built the direction-prediction part of the Pentium M branch predictor: a bimodal table combined with a tagged global table. With four global ways it averages 5.848 mispredictions per 1,000 instructions (MPKI) over the 20 traces that come with the infrastructure. Below I describe how it works, compare it with a few other configurations, and discuss what the numbers show.

AI use: I used an AI assistant (Claude) to help with the code and this report, as permitted for this assignment.

# Assignment 1: Pentium M branch predictor

## How to build and run

cd src

make -B CXXFLAGS='-O3 -Wall'   # builds the predictor on macOS

cd ..

csh ./run traces        # MPKI for every trace and the average

./src/predict traces/164.gzip/gzip.trace.bz2      # a single trace

I built the predictor on macOS with make -B CXXFLAGS='-O3 -Wall', which leaves out the debug symbols. All commands are run from cbp2-infrastructure-v2/.

predict_extra_credit (the complete Pentium M predictor) is still the empty placeholder from the starter code. All results in this report come from pm_predictor.

run is a csh script. If it isn't executable, start it with csh ./run traces, or run chmod +x run first.

The trace reader passes the file name to bzip2 without quotes, so a path containing spaces breaks it. Using relative paths, as above, avoids the problem.

Build options (add them to CXXFLAGS in src/Makefile):

| flag | effect |
| --- | --- |
| -DPM_GLOBAL_WAYS=2 | 2-way global predictor instead of 4-way |
| -DPM_ALLOCATE_ALWAYS | allocate a global entry on every global miss, not only when the bimodal predictor was wrong |

## 1. What I implemented

My code is the pm_predictor class in src/my_predictor.h. It covers the two parts the assignment requires, the bimodal table and the global predictor (the blue blocks in the diagram). The loop predictor, target prediction (BTB/iBTB) and the PIR are optional, and I left them out.

| lookup | input / index | result |
| --- | --- | --- |
| Bimodal | IP[11:0] | Prediction from a two-bit counter |
| Global | HASH = (IP XOR GHR)[14:0]; set = HASH[14:6]; tag = HASH[5:0] | Search four tagged ways |
| Selection | Global tag hit? | Use global on a hit; otherwise use bimodal |

| structure | size | storage |
| --- | --- | --- |
| Bimodal table | 4096 two-bit counters, index = IP[11:0] | 8 Kbit |
| Global predictor | 512 sets × 4 ways; each entry: valid, 6-bit tag, 2-bit counter, LRU | 18 Kbit (+ LRU) |
| Global history register | 15 bits |  |

Prediction (predict), conditional branches only:

1. Read the bimodal counter at IP[11:0].

2. Form HASH = (IP XOR GHR)[14:0], then search the 4 ways of set HASH[14:6] for tag HASH[5:0].

3. On a tag hit, the final prediction is the global counter; otherwise it is the bimodal counter.

Unconditional branches are predicted taken. Target prediction returns 0 (not required).

Update (update), conditional branches only:

1. The bimodal counter is always trained (saturating ±1).

2. On a global hit, the global counter is trained and the way becomes most recently used.

3. On a global miss, a new global entry is allocated only if the bimodal prediction was wrong. It takes an invalid way if there is one, otherwise the LRU way, and its counter starts weakly toward the actual outcome. This follows the published description of the Pentium M: only branches the bimodal table gets wrong use space in the global table.

4. The outcome is shifted into the GHR.

The real Pentium M hashes the IP with a path information register (PIR). Since the PIR is optional here, I used a global history register of conditional-branch outcomes instead, the same GHR-XOR-address idea as the example in Appendix B of the assignment.

I wrote the global table and its LRU replacement myself inside the predictor class, rather than using an existing cache implementation.

## 2. Results

All numbers are direction mispredictions per 1,000 instructions (MPKI). Each trace is 100 million instructions, so MPKI is simply the number of misses divided by 100,000. Averages are taken over the unrounded values for all 20 traces. Columns marked A allocate a global entry on every global miss, instead of only when the bimodal prediction was wrong.

| trace | always taken | bimodal only | gshare 32K (sample) | PM 2-way | PM 2-way A | PM 4-way | PM 4-way A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| gzip | 70.025 | 15.030 | 12.473 | 12.520 | 12.617 | 12.436 | 12.459 |
| vpr | 85.312 | 14.671 | 13.415 | 13.866 | 14.337 | 13.298 | 14.070 |
| gcc | 86.071 | 19.347 | 11.254 | 10.618 | 15.298 | 9.285 | 13.304 |
| mcf | 102.497 | 33.245 | 15.837 | 16.398 | 17.680 | 15.621 | 16.488 |
| crafty | 48.651 | 13.075 | 5.837 | 5.983 | 8.464 | 5.211 | 6.895 |
| parser | 78.421 | 15.417 | 10.008 | 9.036 | 10.357 | 8.538 | 9.242 |
| compress | 76.983 | 9.356 | 7.831 | 7.854 | 8.199 | 7.702 | 7.839 |
| jess | 92.343 | 8.809 | 1.562 | 1.149 | 1.750 | 1.118 | 1.464 |
| raytrace | 109.506 | 3.806 | 2.756 | 2.503 | 3.421 | 2.307 | 2.952 |
| db | 91.451 | 6.480 | 3.909 | 3.723 | 4.613 | 3.615 | 4.283 |
| javac | 109.644 | 3.181 | 2.267 | 2.126 | 2.638 | 2.037 | 2.397 |
| mpegaudio | 110.339 | 3.154 | 2.188 | 2.124 | 2.604 | 2.028 | 2.352 |
| mtrt | 112.022 | 3.706 | 2.657 | 2.470 | 3.268 | 2.383 | 2.957 |
| jack | 77.958 | 7.691 | 3.033 | 2.420 | 3.946 | 2.353 | 3.075 |
| eon | 24.226 | 9.405 | 1.807 | 1.501 | 3.139 | 1.425 | 1.837 |
| perlbmk | 66.833 | 5.955 | 2.554 | 1.917 | 4.700 | 1.819 | 3.279 |
| gap | 75.950 | 9.675 | 3.926 | 3.604 | 4.797 | 3.507 | 3.814 |
| vortex | 70.524 | 1.916 | 1.222 | 0.854 | 1.239 | 0.828 | 1.085 |
| bzip2 | 178.257 | 0.112 | 0.094 | 0.059 | 0.076 | 0.057 | 0.073 |
| twolf | 59.221 | 21.942 | 21.489 | 21.981 | 22.046 | 21.389 | 22.236 |
| average | 86.312 | 10.299 | 6.306 | 6.135 | 7.260 | 5.848 | 6.605 |

I used four global ways for the main configuration. Here is part of the output saved in results_pm4.txt:

traces/164.gzip/gzip.trace.bz2              12.436

traces/175.vpr/vpr.trace.bz2                13.298

traces/176.gcc/gcc.trace.bz2                9.285

...

traces/300.twolf/twolf.trace.bz2            21.389

average MPKI: 5.847

The run script cuts its average off at three decimals, which is why it prints 5.847. The actual average is 5.8478665, so the table shows 5.848. In the same way, the PM 2-way A average of 7.2596035 rounds to 7.260.

## 3. Discussion

Compared with the bimodal table alone (10.299 MPKI), the four-way hybrid brings the average down to 5.848, about 43% fewer mispredictions. It also beats the sample gshare predictor (6.306 MPKI) by about 7%. I didn't test the tagged global table on its own, so I can't say how much of the gain it would give by itself.

For storage, the bimodal counters take 8 Kbit and the four-way global table another 18 Kbit for valid bits, tags and counters, not counting history and LRU state. (The C++ version uses more memory than that, since each counter is a whole byte and each entry has an integer LRU stamp.) The tags keep different branches apart within a set, but two PC/history combinations can still hash to the same value.

Going from two ways to four lowers the average from 6.135 to 5.848. The biggest gains are on gcc (10.618 to 9.285) and crafty (5.983 to 5.211). Both versions have 512 sets, though, so the four-way table also has twice as many entries; part of the improvement comes from the extra capacity, not just the higher associativity.

The allocation policy made a clear difference. With four ways, allocating on every global miss raises the average from 5.848 to 6.605. My guess is that allocating only after a bimodal miss keeps the table free for the branches that actually need it, but I didn't count evictions, so I haven't confirmed this.

twolf, mcf, vpr and gzip stay above 12 MPKI with every predictor I tried. The totals alone don't explain why; separating the effects of capacity, aliasing, loop behaviour or path history would take more experiments.

## Source: pm_predictor (src/my_predictor.h)

//

// Pentium M hybrid branch predictors

// This class implements a simple hybrid branch predictor based on the Pentium M branch outcome prediction units.

// Instead of implementing the complete Pentium M branch outcome predictors, the class below implements a hybrid

// predictor that combines a bimodal predictor and a global predictor.

//

// Structure (see the Pentium M diagram in the assignment):

//   - Bimodal table: 4096 2-bit counters, index = IP[11:0].

//   - Global predictor: 512 sets x PM_GLOBAL_WAYS ways. Each entry has a

//     valid bit, a 6-bit tag, a 2-bit counter and an LRU stamp.

//     HASH = IP[14:0] XOR GHR[14:0]; index = HASH[14:6], tag = HASH[5:0].

//     (The real Pentium M hashes the IP with a path information register;

//     the assignment lets us skip the PIR, so a global history register of

//     conditional-branch outcomes is used instead.)

//   - Final prediction: global prediction on a tag hit, bimodal otherwise.

//

// Update policy:

//   - The bimodal counter is always trained.

//   - On a global hit the global counter is trained and the way becomes MRU.

//   - On a global miss a new entry is allocated (LRU victim) only when the

//     bimodal predictor was wrong, so the global table holds only branches

//     the bimodal table cannot handle. Define PM_ALLOCATE_ALWAYS to allocate

//     on every miss instead (the policy used in the Appendix B example).

#ifndef PM_GLOBAL_WAYS

#define PM_GLOBAL_WAYS      4   // the assignment also allows 2

#endif

#define PM_BIMODAL_BITS     12  // 4096 entries

#define PM_GLOBAL_INDEX_BITS    9   // 512 sets

#define PM_GLOBAL_TAG_BITS  6

#define PM_HASH_BITS        (PM_GLOBAL_INDEX_BITS + PM_GLOBAL_TAG_BITS) // 15

struct pm_global_entry {

bool valid;

unsigned char tag;

unsigned char ctr;      // 2-bit saturating counter

unsigned int lru;       // larger = more recently used

};

class pm_update : public branch_update {

public:

unsigned int index;     // bimodal index

unsigned int set;       // global set

unsigned int tag;       // global tag

int way;            // hit way, -1 on a global miss

bool bimodal_prediction;

};

class pm_predictor : public branch_predictor {

public:

pm_update u;

branch_info bi;

unsigned int history;

unsigned int lru_clock;

unsigned char bimodal[1<<PM_BIMODAL_BITS];

pm_global_entry global[1<<PM_GLOBAL_INDEX_BITS][PM_GLOBAL_WAYS];

pm_predictor (void) : history(0), lru_clock(0) {

// counters start weakly not-taken; the global table starts empty

memset (bimodal, 1, sizeof (bimodal));

memset (global, 0, sizeof (global));

}

branch_update *predict (branch_info & b) {

bi = b;

if (b.br_flags & BR_CONDITIONAL) {

// bimodal lookup

u.index = b.address & ((1<<PM_BIMODAL_BITS)-1);

u.bimodal_prediction = bimodal[u.index] >> 1;

// global lookup

unsigned int hash = (b.address ^ history) & ((1<<PM_HASH_BITS)-1);

u.set = hash >> PM_GLOBAL_TAG_BITS;

u.tag = hash & ((1<<PM_GLOBAL_TAG_BITS)-1);

u.way = -1;

for (int w = 0; w < PM_GLOBAL_WAYS; w++) {

pm_global_entry &e = global[u.set][w];

if (e.valid && e.tag == u.tag) {

u.way = w;

break;

}

}

// global hit overrides the bimodal prediction

if (u.way >= 0)

u.direction_prediction (global[u.set][u.way].ctr >> 1);

else

u.direction_prediction (u.bimodal_prediction);

} else {

u.direction_prediction (true);

}

// branch target prediction is not required

u.target_prediction (0);

return &u;

}

void update (branch_update *bu, bool taken, unsigned int target) {

if (!(bi.br_flags & BR_CONDITIONAL)) return;

pm_update *p = (pm_update *) bu;

// bimodal table is always trained

train (bimodal[p->index], taken);

if (p->way >= 0) {

pm_global_entry &e = global[p->set][p->way];

train (e.ctr, taken);

e.lru = ++lru_clock;

} else {

#ifndef PM_ALLOCATE_ALWAYS

if (p->bimodal_prediction != taken)

#endif

allocate (p->set, p->tag, taken);

}

history = ((history << 1) | taken) & ((1<<PM_HASH_BITS)-1);

}

// replace an invalid way if there is one, otherwise the LRU way

void allocate (unsigned int set, unsigned int tag, bool taken) {

int victim = 0;

for (int w = 0; w < PM_GLOBAL_WAYS; w++) {

if (!global[set][w].valid) { victim = w; break; }

if (global[set][w].lru < global[set][victim].lru) victim = w;

}

pm_global_entry &e = global[set][victim];

e.valid = true;

e.tag = tag;

e.ctr = taken ? 2 : 1;      // weakly toward the actual outcome

e.lru = ++lru_clock;

}

static void train (unsigned char &c, bool taken) {

if (taken) {

if (c < 3) c++;

} else {

if (c > 0) c--;

}

}

};